// THE ROBOTIC FLOWER

// Code put together by Inga Woods-Waight

// TikTok - @ingawoods.waight

// Instagram - @ingawoods.waight

#include <Wire.h>

#include <Adafruit_PWMServoDriver.h>

// constants won't change

const int TRIG_PIN = 6;              // Arduino pin connected to Ultrasonic Sensor's TRIG pin

const int ECHO_PIN = 7;              // Arduino pin connected to Ultrasonic Sensor's ECHO pin

const int DISTANCE_THRESHOLD = 150;  // centimeters

// called this way, it uses the default address 0x40

Adafruit_PWMServoDriver pwm = Adafruit_PWMServoDriver();

#define SERVOMIN 150   // This is the 'minimum' pulse length count (out of 4096)

#define SERVOMAX 600   // This is the 'maximum' pulse length count (out of 4096)

#define USMIN 600      // This is the rounded 'minimum' microsecond length based on the minimum pulse of 150

#define USMAX 2400     // This is the rounded 'maximum' microsecond length based on the maximum pulse of 600

#define SERVO_FREQ 50  // Analog servos run at ~50 Hz updates

// variables will change:

float duration_us, distance_cm;

void setup() {

Serial.begin(9600);

pinMode(TRIG_PIN, OUTPUT);  // set arduino pin to output mode

pinMode(ECHO_PIN, INPUT);   // set arduino pin to input mode

Serial.println("4 channel Servo test!");

pwm.begin();

pwm.setPWMFreq(SERVO_FREQ);  // Analog servos run at ~50 Hz updates

}

void loop() {

int pos = 125;

int pos_m = 485;

// generate 10-microsecond pulse to TRIG pin

digitalWrite(TRIG_PIN, HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);

// measure duration of pulse from ECHO pin

duration_us = pulseIn(ECHO_PIN, HIGH);

// calculate the distance

distance_cm = 0.017 * duration_us;

if (distance_cm < DISTANCE_THRESHOLD)

close();  // rotate servo motor to 0 degree

else

open();  // rotate servo motor to 180 degree

// print the value to Serial Monitor

Serial.print("distance: ");

Serial.print(distance_cm);

Serial.println(" cm");

// delay(500);

}

void open() {

int pos = 125;

for (int pos = 125; pos < 485; pos += 10)

;

{  // Loop with movement slow from 30 degrees to 150 degrees

pwm.setPWM(0, 0, pos);

pwm.setPWM(1, 0, pos);

pwm.setPWM(2, 0, pos);

pwm.setPWM(3, 0, pos);

Serial.println("0");

delay(50);

}

delay(5000);

}

void close() {

int pos_m = 485;

for (int pos_m = 485; pos_m > 125; pos_m -= 10)

;

{  // Loop with movement slow from 150 degrees to 30 degrees

pwm.setPWM(0, 0, pos_m);

pwm.setPWM(1, 0, pos_m);

pwm.setPWM(2, 0, pos_m);

pwm.setPWM(3, 0, pos_m);

Serial.println("180");

delay(50);

}

delay(5000);

}. check this code, i am having a problem with the timing , i want the sensor to detect when someone move close. why is the sensor slow in responding